Portable Prompter Setup

- Loosen the two thumb screws on base of Impact – Heavy Duty Tripod. This will allow you to expand the tripod legs.

- After expanding tripod legs, tighten both base thumb screws. Ensure there is enough clearance at the base of the tripod to fit the sand bag.

- Insert sand bag at bottom of tripod base. This will ensure the prompter will not fall due to inclement weather.

- Loosen lower extendable center thumb and extend the height to about chin level.

- Next Screw the double-ended spigot tightly into the bottom of the portable prompter.

- Loosen the top Thumb screw on the extendable center.

- Attach portable prompter to the tripod and tighten the top thumb screw

- Loosen thumb screw on the back of the Prompter to adjust the mirror.

- Adjust mirror (top) at a 45 degree angle from the prompter screen (bottom) and then tighten the Thumb screws on the back of the prompter.

- Next locate the magnetic prompter hood, Unfold and zip up both sides of the hood.

- Place the magnetic hood onto the back of the mirror (top).

- Connect the power supply (screw in to secure) and HDMI cable into the back of the prompter.

- If you are shooting inside and have access to a power outlet. Connect the power cable to outlet.

13a. If you do not have access to power. Connect prompter power to the Portable power supply in the Prompter kit.

- Next Connect HDMI cable to your Laptop.

14a. If you do not have an HDMI port on your laptop. Use the USB-C to HDMI cable provided.

- Locate Thumb mouse and plug usb dongle into your laptop.

- Power on portable power supply if you are not connected to power outlet. Then Power on Prompter

- Adjust Screen Resolution for Prompter to 1280 x 1024

  - Right click on Desktop and select Display Settings

  - Press Identify to find the screen number for the portable prompter. Select Screen 2 (it should be highlighted blue)

  - Scroll down in your display setting to find “Display resolution”. Click the drop down arrow and select 1280 x 1024

  - When Prompted  to keep display settings. Select “Keep Changes”

  - Exit out of display settings

- Launch Prompter4 Program with Script (see KNPN How To’s – Prompter4  for more information)

- Drag talent screen to Prompter screen and maximize window

- Ensure there is a battery in the thumb mouse. Turn on and scroll up/down thru script.

Scroll

Power